HOBP type 5

## Introductions

| Version 1.0 |  | …. |
| --- | --- | --- |
| Version 2.0 (CT) | Key2go process can be skipping for Key Type. These modifications result from we found that the number of the sequence is limited at 16 different sequences for Key type. | 17 JULY 2014 |
| Version 2.1 (CT) | Remove the exception response for the version 2. Need two log challenge-response into the learning process (getPolynome()) | 18 JULY 2014 |

Link svn: svn://eng/svn.datastart/lpc2300/core/code/car_libs/lib.hobp_type5_v2

Files: “hobp_type5.c”, “hobp_private.h”

This document gives some information about the Bypass Honda Type 5 library (Hobp) version 2.

## Initializations

To initializes the library, call the function: initBypassLib(void). This function returns nothing.

## Get the table 1

To process the challenges to get the Data use to build Table 1, call the function:

UInt8  getTable1    ( UInt8  * u8Challenge    , UInt8 * u8Response, UInt8 * u8Tab1Col1, UInt8 (*u8Tab1)[16]  );

This function receives the response of the previous challenge, checks is the response is correct, saves the response and gives back the next challenge to process. Theses responses are use to create the table 1.  76 challenges if there are not bad response.

* Parameters: - UInt8  * u8Challenge :Pointer of Table unsigned char of 3 values. Next challenge to process.

- UInt8 * u8Response :  Pointer of Table unsigned char of 3 values. Response of the previous challenge.

- UInt8 * u8Tab1Col1 :   Pointer of Table unsigned char of 16 values.  Column 1 of the Table 1.

Uses to create the Table 1

- UInt8* (*u8Tab1)[16] :   Pointer of Table dimension 2 UInt8  [4][16].

4 difference line of the Table1. Uses to create the Table 1

* Return Value: - Type UInt8

0 : Processing

1 : Procces is done

2 : Error, All retry challenges were tested

## Set the table of the sequence 1

To build the table of sequence (iTabStatus, u8TabSeq) depending of the type define (HONDA_T5_KEY, HONDA_T5_PTS), call the function:

void setTableSeq(void);

## Learning Process to get the polynomial number

To process the learning and get the polynomial number, call the function:

int		getPolynome( const UInt8 * u8Challenge, const UInt8 * u8Response, const UInt8 * u8Challenge2, const UInt8 * u8Response2, int iMMOStatus);

This function will set the global variable u8IndexeSeq. This variable is use to point to the right sequence. Set the polynomial number UInt8 u8Poly [2].

* Parameters: - const UInt8 * u8Challenge

- const UInt8 * u8Response

- const UInt8 * u8Challenge2

- const UInt8 * u8Response2

- int iMMOStatus (example : 0x55888088)

* Return Value: - Type int

0 : OK

-1 : Error

## Get Response

When the learning process is done, all data are set.  So now, we can process, Challenge – Response algorithm.

To get a response depending of a specific challenge, call the function:

void    algo_Honda_V2(const UInt8 * cChallenge, UInt8 * cResponse)

* Parameters: - const UInt8 * u8Challenge

- UInt8 * u8Response